APPENDIX A

Expanded Block Diagram of the Face Detection System

APPENDIX B

SOCKIT DEVELOPMENT KITS & TOOLS DETAIL

| Description | The SoCKit Development Kit presents a robust hardware design platform built around the Altera System-on-Chip (SoC) FPGA, which combines the latest dual-core Cortex-A9 embedded cores with industry-leading programmable logic for ultimate design flexibility. Users can now leverage the power of tremendous re-configurability paired with a high-performance, low-power processor system. Altera’s SoC integrates an ARM-based hard processor system (HPS) consisting of processor, peripherals and memory interfaces tied seamlessly with the FPGA fabric using a high-bandwidth interconnect backbone. The SoCKit development board includes hardware such as high-speed DDR3 memory, video and audio capabilities, Ethernet networking, and much more. In addition, an on-board HSMC connector with high-speed transceivers allows for an even greater array of hardware setups. By leveraging all of these capabilities, the SoCKit is the perfect solution for showcasing, evaluating, and prototyping the true potential of the Altera SoC. |
| --- | --- |
| Features | FPGA Device • Cyclone V SoC 5CSXFC6D6F31 Device • Dual-core ARM Cortex-A9 (HPS) • 110K Programmable Logic Elements • 5,140 Kbits embedded memory • 6 Fractional PLLs • 2 Hard Memory Controllers • 3.125G Transceivers Configuration and Debug • Quad Serial Configuration device – EPCQ256 on FPGA • On-Board USB Blaster II (micro USB type B connector) Memory Device • 1GB (2x256MBx16) DDR3 SDRAM on FPGA • 1GB (2x256MBx16) DDR3 SDRAM on HPS • 128MB QSPI Flash on HPS • Micro SD Card Socket on HPS Communication • USB 2.0 OTG (ULPI interface with micro USB type AB connector) • USB to UART (micro USB type B connector) • 10/100/1000 Ethernet Connectors • One HSMC (8-channel Transceivers, Configurable I/O standards 1.5/1.8/2.5/3.3V) • One LTC connector (One Serial Peripheral Interface (SPI) Master ,one I2C and one GPIO interface ) Display • 24-bit VGA DAC • 128x64 dots LCD Module with Backlight Audio • 24-bit CODEC, Line-in, line-out, and microphone-in jacks Switches, Buttons and LEDs • 8 User Keys (FPGA x4 ; HPS x 4) • 8 User Switches (FPGA x4 ; HPS x 4) • 8 User LEDs (FPGA x4 ; HPS x 4) • 2 HPS Reset Buttons (HPS_RSET_n and HPS_WARM_RST_n) Sensors • G-Sensor on HPS • Temperature Sensor on FPGA Power • 12V DC input |
| Kit Includes | • The SoCKit development board • USB Cable for FPGA programming and control • Ethernet Cable • 12V DC power adapter |

APPENDIX C

ARM® Cortex™-A9 MPCore™ Specification

- 800 MHz dual-core processor supporting symmetric and asymmetric multiprocessing - Each processor includes the following: - High-efficiency, dual-issue superscalar pipeline (2.5 MIPS* per MHz) - NEONTM media processing engine for media and signal processing acceleration - Single- and double-precision floating-point unit - 32 KB instruction and 32 KB data caches - Cache coherence for enhanced inter-processor communication - Memory Management Unit with TrustZone® security technology - Thumb®-2 technology for enhanced code density, performance, and power efficiency - Jazelle® architecture extensions for accelerating Java Virtual Machine - Program Trace Macrocell for full visibility of processor instruction flow - Shared 512 KB, 8-way associative L2 cache, lockable by way, line, or master - Acceleration coherency port that extends coherent memory access beyond the CPUs - Generic interrupt controller - 32 bit general purpose timer - Watchdog timer - Available in Altera® Arria® V SoCs and Cyclone® V SoCs

APPENDIX D

FPGA Configuration Mode (Taken from SoCKit User Manual)

APPENDIX D (CONTINUED)

FPGA Configuration Mode (Taken from SoCKit User Manual)

APPENDIX D (CONTINUED)

FPGA Configuration Mode (Taken from SoCKit User Manual)

APPENDIX E

Configuring U-Boot to the Proper Locations of Kernel Program

bootcmd=run mmcload; run fpgaload; run mmcboot

fpga=0

fpgadata=0x2000000

fpgadatasize=2126D5

fpgaload=fatload mmc 0:1 0x2000000 soc_system.rbf; fpga load 0 ${fpgadata} ${fi}

Configuring the Serial Console to Port 57600 for Debugging

setenv mmcboot 'setenv bootargs console=tty0 console=ttyS0,57600 root=${mmcroot} rw rootwait;bootm ${loadaddr} - ${fdtaddr}'

APPENDIX F

From RocketBoards Org On How to Partition a uSD Card.

#### Partition the SD Card

- Determine the device associated with the SD card on the host by running the following command before and after inserting the card in the reader:

$ cat /proc/partitions

Let's assume it is /dev/sdx

- Repartition the card, with the sudo fdisk/dev/sdx command

$ Use p to print current partitioning

$ Use d to delete all partitions one by one

Create partition using these options, pressing enter after each one: n p 2 4096 +4496384 t 83
Create partition using these options, pressing enter after each one: n p 1 9000000 +20480K t 1 b
Create partition using these options, pressing enter after each one: n p 3 2048 +1024K t 3 a2

- Press w to write partitions to card, then exit

Verify the card was partitioned correctly, it should look like this (use fdisk and p to see it): 
Device Boot Start End Blocks Id System 
/dev/sdx1 9000000 9020480 20480 b W95 FAT32 
/dev/sdx2 4096 8996863 4496384 83 Linux 
/dev/sdx3 2048 4095 1024 a2 Unknown

- Make partition one a DOS partition

$ sudo mkdosfs /dev/sdx1

APPENDIX G

Specs of the Intel CISC based system

Host Name: ANDROMEDA OS Name: Microsoft Windows 7 Ultimate OS Version: 6.1.7601 Service Pack 1 Build 7601 OS Manufacturer: Microsoft Corporation OS Configuration: Standalone Workstation OS Build Type: Multiprocessor Free Registered Owner: Bangonkali Registered Organization: Product ID: 00426-OEM-8992662-00006 Original Install Date: 11/30/2013, 1:30:10 AM System Boot Time: 3/13/2014, 9:30:26 PM System Manufacturer: Acer System Model: Aspire 4755 System Type: x64-based PC Processor(s): 1 Processor(s) Installed. [01]: Intel64 Family 6 Model 42 Stepping 7 GenuineInt el ~792 Mhz BIOS Version: Phoenix Technologies Ltd. V2.13, 10/21/2011 Windows Directory: C:\Windows System Directory: C:\Windows\system32 Boot Device: \Device\HarddiskVolume1 System Locale: en-us;English (United States) Input Locale: en-us;English (United States) Time Zone: (UTC+08:00) Taipei Total Physical Memory: 8,003 MB Available Physical Memory: 4,280 MB Virtual Memory: Max Size: 16,005 MB Virtual Memory: Available: 12,066 MB Virtual Memory: In Use: 3,939 MB Page File Location(s): C:\pagefile.sys Domain: WORKGROUP Logon Server: \\ANDROMEDA

APPENDIX H

Kernel Panic Logs on Continue Long Term Operation of Face Detection System

Unable to handle kernel NULL pointer dereference at virtual address 0000003c pgd = 80004000 [0000003c] *pgd=00000000 Internal error: Oops: 17 [#1] SMP ARM Modules linked in: ipv6 uvcvideo videobuf2_core videodev videobuf2_vmalloc video buf2_memops gpio_dw CPU: 0 Not tainted (3.8.0-00111-g85cc90f #1) PC is at usb_hcd_unmap_urb_setup_for_dma+0x8/0xb0 LR is at usb_hcd_unmap_urb_for_dma+0x14/0x134 pc : [<8029b110>] lr : [<8029b1cc>] psr: a0000193 sp : bed8bc88 ip : 00000000 fp : 00000001 r10: be11b0d0 r9 : 80559a34 r8 : 00000000 r7 : 00000018 r6 : 000000a0 r5 : be11b000 r4 : 00000000 r3 : bc194b08 r2 : 800400c7 r1 : 00000000 r0 : be11b000 Flags: NzCv IRQs off FIQs on Mode SVC_32 ISA ARM Segment user Control: 10c5387d Table: 3c01c04a DAC: 00000015 Process facedetect (pid: 1646, stack limit = 0xbed8a240) Stack: (0xbed8bc88 to 0xbed8c000) bc80: 00000000 be11b000 000000a0 8029b1cc be11b0fc be11b104 bca0: be010a10 bebf2280 0000000c 000000a0 00000018 802b2738 bed8bd14 00001000 bcc0: 00000081 80546800 00000001 00001000 805467e8 8024db6c 0000004c 00000033 bce0: 20b48bfc bebf2280 c0d805e0 802afd08 8001c960 00000001 000003fc 00000040 bd00: 00000000 bebf2280 00000000 00000000 0000b071 60000193 be010a10 000000a0 bd20: 00000000 00000000 80559a34 80559a20 00000000 802af9f8 802af9ec 8029ab1c bd40: bebf1100 80075bf0 00000001 bebf2ec0 00000002 be0109c0 bed8bdb4 be0109c0 bd60: be010a10 00000000 fee00100 be11b0d0 00000000 fffffffe a0000013 80075e24 bd80: be0109c0 be010a10 00000000 80078bac 80078b30 000000a0 000000a0 800755d4 bda0: 80525030 8000f014 fee0010c 8052e418 bed8bdd0 80008530 802b070c 803b6a88 bdc0: 60000013 ffffffff bed8be04 8000dd40 be11b0d0 a0000013 00000000 8a258a25 bde0: bedb4e00 be5f7000 bc194b00 be11b000 be11b0d0 00000000 fffffffe a0000013 be00: 0000003d bed8be18 802b070c 803b6a88 60000013 ffffffff 802b0644 fffffffe be20: be5f7000 be11b000 fffffffe be419000 be31b788 00000001 bed8a000 8029b3f8 be40: be1c0400 fffffffe be5f7000 60000013 00000000 be419000 00000001 8029c708 be60: be5f7000 be5f700c 00000001 8029d590 bed8beac be41900c be5f6000 00000002 be80: 00000000 be419000 be31b788 be419010 be5f7000 7f041a68 00000001 be419000 bea0: 00000000 00000000 be419000 00000008 00000000 7f043e54 00000001 be43bd80 bec0: be31b780 bd11ea18 be419000 7f03ff78 7f03fee0 be23fc00 be664e88 bd11ea18 bee0: be05b310 7f014390 7f014358 be31b780 be664e88 800db0d8 00000000 00000000 bf00: be2d283c beba66c0 805618c4 be2d2580 8000e2a8 bed8a000 00000000 8003e488 bf20: be1e68c0 fd87fd87 be2d2580 be1e68c0 be2d284c be2d2580 be1e68fc 800265c8 bf40: be1e68c0 00000000 bed8a000 00000001 00382000 800c4960 fd87fd87 800da1ac bf60: be1e6900 be57f680 00000000 bed8a000 000000f8 8000e2a8 bed8a000 00000000 bf80: 00000000 80026d88 00000000 000703c2 7698e760 7698e760 000000f8 80026e2c bfa0: 00000000 8000e100 000703c2 7698e760 00000000 000703ae 74c214c0 00000000 bfc0: 000703c2 7698e760 7698e760 000000f8 00000000 00000000 76fa1000 00000000 bfe0: 000000f8 7eab82bc 7691fce3 768c71e6 600f0030 00000000 00000000 00000000 [<8029b110>] (usb_hcd_unmap_urb_setup_for_dma+0x8/0xb0) from [<be11b104>] (0xbe1 1b104) Code: 8055994b 80497e64 e92d4070 e1a04001 (e591303c) ---[ end trace 7240ef6d486fb03d ]---

APPENDIX I

Researcher’s Notes on SoCKit Usage

Sockit usage notes: As root: sudo chmod u+s /usr/bin/sudo su linaro sudo apt-get update && \ sudo apt-get upgrade && \ sudo ntpdate time.nist.gov && \ sudo apt-get install wget build-essential checkinstall libncurses5-dev libncurses5 libevent-1.4 libevent-dev libncurses5-dev pkg-config libpng12-0 libpng12-dev libpng++-dev libpng3 libpnglite-dev zlib1g-dbg zlib1g zlib1g-dev pngtools libtiff4-dev libtiff4 libtiffxx0c2 libtiff-tools ffmpeg v4l-conf v4l-utils v4l2ucp libjpeg-dev libpng-dev libtiff-dev libjasper-dev libavcodec-dev libavformat-dev libswscale-dev usbutils gparted vim less psmisc lynx gcc g++ xserver-xorg xterm firefox mplayer lxde xinit x11-apps xinput-calibrator mesa-utils prboom Check webcam v4l2-ctl --list-formats v4l2-ctl --list-formats-ext v4l2-ctl --set-fmt-video=width=1280,height=720,pixelformat=1 v4l2-ctl --set-fmt-video=width=640,height=480,pixelformat=1 v4l2-ctl --set-parm=30 lsusb 1. Find & install CMAKE download cmake http://www.cmake.org/cmake/resources/software.html tar xf cmake* cd cmake* ./bootstrap && make && sudo make install Installing Synergy sudo apt-get install xorg-dev libqt4-dev python libcurl4-openssl-dev [cant be detected] python-dev make gcc http://synergy-foss.org/wiki/Compiling ./hm.sh conf -g1 --make-gui ./hm.sh build --make-gui HTOP svn co https://htop.svn.sourceforge.net/svnroot/htop/trunk htop --disable-unicode Installing OpenSSH http://www.unixwiz.net/techtips/openssh.html#tarballs ./configure --with-ipv4-default --with-md5-passwords --with-pam && make && make install ssdh privilege escalation groupadd sshd useradd -M -g sshd -c 'sshd privsep' -d /var/empty -s /sbin/nologin sshd passwd -l sshd useradd -g root linaro building cmake http://www.cmake.org/cmake/help/install.html http://docs.opencv.org/doc/tutorials/introduction/linux_install/linux_install.html sudo apt-get install ffmpeg && \ sudo apt-get install v4l-conf v4l-utils v4l2ucp && \ sudo apt-get install libjpeg-dev libpng-dev libtiff-dev libjasper-dev && \ sudo apt-get install libavcodec-dev libavformat-dev libswscale-dev time sudo apt-get install ntp Probably the ntp service is running, that's why ntpdate can't open the socket (port 123 UDP) and connect to ntp server. Try from command line: sudo service ntp stop sudo ntpdate -s time.nist.gov sudo service ntp start If you want to put this in /etc/rc.local use the following: ( /etc/init.d/ntp stop until ping -nq -c3 8.8.8.8; do echo "Waiting for network..." done ntpdate -s time.nist.gov /etc/init.d/ntp start )& http://askubuntu.com/questions/254826/how-to-force-a-clock-update-using-ntp https://help.ubuntu.com/10.04/serverguide/NTP.html chmod -R 777 /tmp chmod -R 777 /usr/local/src chown -R root:root /usr/bin/sudo chmod 4755 /usr/bin/sudo apt-get update apt-get dist-upgrade apt-get install wget #!/bin/bash cd /usr/local/src //zlib wget http://zlib.net/zlib-1.2.8.tar.gz wget http://www.openssl.org/source/openssl-1.0.1f.tar.gz wget ftp://ftp3.usa.openbsd.org/pub/OpenBSD/OpenSSH/portable/openssh-6.4p1.tar.gz tar -xf openssh-6.4p1.tar.gz tar -xf openssl-1.0.1f.tar.gz tar -xf zlib-1.2.8.tar.gz cd /usr/local/src cd zlib-1.2.8 ./configure && make && sudo make install cd ../openssl-1.0.1f ./config && make && make test && sudo make install cd ../openssh-6.4p1 ./configure --with-ipv4-default --with-md5-passwords && make sudo groupadd sshd sudo useradd -M -g sshd -c 'sshd privsep' -d /var/empty -s /sbin/nologin sshd sudo passwd -l sshd sudo make install // set time http://askubuntu.com/questions/254826/how-to-force-a-clock-update-using-ntp apt-get install ntp service ntp start sudo service ntp stop sudo ntpdate -s time.nist.gov sudo service ntp start //ssh http://www.unixwiz.net/techtips/openssh.html sshd re-exec requires execution with an absolute path // install x11 // install arm compiler sudo apt-get install libc6-dev-i386 download source compile source http://askubuntu.com/questions/251978/cannot-find-crti-o-no-such-file-or-directory apt-get install libc-dev export LIBRARY_PATH=/usr/lib32:/usr/lib/x86_64-linux-gnu export LIBRARY_PATH=/usr/lib/x86_64-linux-gnu/ http://askubuntu.com/questions/98416/error-kernel-headers-not-found-but-they-are-in-place sudo apt-get install linux-headers-$(uname-r) http://stackoverflow.com/questions/11471722/libstdc-so-6-cannot-open-shared-object-file-no-such-file-or-directory sudo apt-get install lib32z1 sudo apt-get install lib32z1 lib32z1-dev sudo apt-get install lib32stdc++6 sudo apt-get install ia32-libs sudo apt-get install uboot-mkimage CHANGING IMAGE on Boot http://sumanprasanna.wordpress.com/2012/12/11/how-to-change-your-boot-logo-in-linux-kernel/

APPENDIX J

Face Detection System SoCKit Test Results

APPENDIX K

Standard 512x512 pixels test image.

APPENDIX L

Intel CISC Based Core i7 Test Results